# RINEX VERIFICATION – Help

This script will compare RINEX processed data and TRINAV/SPN logged data to produce difference plots for Lat and Lon, in metres.

The user selects TRINAV/SPN in Navigation System. The NAVIGATION logged file will be recorded in TRINAV/SPN. The RINEX file will be recorded in FUGRO/VERIPOS systems and will be processed before entering in this script.

The names of the GPS systems have to match those of the headers of the file logged in TRINAV/SPN

GPS system

Navigation Logged

Rinex processed

- Log RINEX data from LD-6 / Verify PC, Fugro, etc.

  - If needed convert the data to RINEX 2.1 format from Veripos or Fugro. Follow Veripos/Fugro work instructions, using conversion software provided by them.

  - Submit the .??O files to https://webapp.geod.nrcan.gc.ca/geod/tools-outils/ppp.php?locale=en

    - log in or make an account if you do not have one.

    - Navigate to the “Geodetic tools and data” page, then to the CSRS-PPP page.

    - Select “Kinematic” and “ITRF”.

    - Load your RINEX files as instructed.

    - Await email with results and download.

  - Use the .csv files processed by Canadian PPP site as input to the RINEX Comparison program

- Log TRINAV

  - Please ensure that the data set does not cross over midnight.

  - line in RT while logging the RINEX data. From QCPR :

- Log SPN

  - Please ensure that the data set does not cross over midnight.

  - If Using SPN, log as follows:

- All antenna offsets need to be set to be zero in both the SPN and the GNSS configurations. In SPN Survey Control create a user output as shown in the screen shots below.

- SPN outputs GPS time from the Raw DGPS Observations. The GPS time field is “Seconds of Week” with the week being the “GPS Week mod 1024”.

- The duration has been set to 2 hours logging time at 1s output rate to match the logging of the GNSS RINEX files.

- The output positions must be RAW.

- Check/Modify Comments  : Scope of Work section. This is output to the report. The default comments can be modified.

- Check Results  : PDF report with time tag suffix is produced on the same directory where the .exe file is located

------Bugs, suggestions, testing datasets, etc, please submit feedback or a ticket referencing SWConnect_Content-7291937 - RINEX Verification -------